# NBCUniversal Regional Sports Networks Terrestrial Handoff Specifications (all Networks except NBC Sports P+)

| Virtual Channel 1 |  |
| --- | --- |
| PID | Value |
| Video | 100 |
| Audio 1 | 101 |
| Audio 2 | 102 |
| DPI | 120 |
| DTMF | 121* Start 121# Stop |

Delivery Options:

- Direct IP connection to NBCUniversal at either/both Englewood Cliffs, NJ or Littleton, CO facilities

  - Requires IP input capable IRDs or other devices capable of ingesting an IP transport stream

  - Contact NBCU Distribution Support (Affiliate Operations) for list of current available fiber vendors at our facilities

- SRT secure Internet Delivery

  - At least one Static Public IP address required from MVPD for NBCU firewall approval

  - Firewall access for NBCU SRT broadcasters on MVPD network

    - Contact NBCU Distribution Support (Affiliate Operations) for list of NBCU public IP addresses as well as traffic ports required for network reception and firewall recommendations

  - SRT enabled receiver

    - MPEG-4 HD Compatible

Delivery Format:

Single Program Transport Streams (SPTS)

Video Specifications:

| Video Codec | Video Bitrate (CBR) | Resolution | Framerate |
| --- | --- | --- | --- |
| H.264 (MPEG-4 HD) | 10 Mbps | 1920 x 1080i | 29.97fps |

Audio Specifications:

| Content | Audio Codec | Audio Bitrate (CBR) | Channels | Audio Sample Rate |
| --- | --- | --- | --- | --- |
| Program Audio | AC3 | 384 Kbps | 5.1 | 48.0 kHz |
| SAP | AC3 | 192 Kbps | 2.0 | 48.0 kHz |

In-Band Metadata:

- Closed Captioning: 608/708

- SCTE 35 (local ad: splice_request, 8s pre-roll for splice start, DTMF)

- Nielsen Watermark

Required bitrate per SPTS (accounts for overhead): 15Mbps

FEC: None

NBCU Distribution Support (Affiliate Operations) Contact info:

201-735-3700

Affiliate.operations@nbcuni.com