SES 1Transponder 23

Technical Parameters

| Satellite: | SES1 |
| --- | --- |
| Orbital Location: | 101 Degrees West |
| Downlink: | 4.16 GHz |
| L-band Frequency: | 1270.00 MHz |
| Transponder: | 23 |
| Transponder Downlink Polarization: | Vertical |
| Symbol Rate: | 30.0000 Msps |
| FEC: | Auto (5/6) |
| Variable Bit Rate Range (all services): | 2 Mbps – 17 Mbps |
| IRD Type: | Cisco PowerVu Model D9859/D9858/D9854/D9824/D9800 |
|  | Addressable/Encrypted |
| HD Compression: SD Compression: HD to SD Downconversion - (CVBS/Transcoded Output): | MPEG-4/DVB-S2 8PSK Compliant MPEG-2/DVB-S2 8PSK Compliant A/R Conversion: 4:3 Auto AFD |
| Program Video & Audio | Refer to table below (on page 2) |
| Network ID: | 9004 |
| Cue Triggers: | Yes, SCTE 35, DTMF |
| Pre-roll: | 8 seconds |
| Local Avail ON: | Yes |
| AUTHORIZATION & TECHNICAL ASSISTANCE |  |
| NBCU Distribution Support | 201-735-3700 |

SES 1 Transponder 23

Technical Parameters

(Cont’d)

Virtual Channel Assignments

| Alt SD |  |  | USA East HD |  |  |  | SyFy East |  |  |  |  |  | Bravo East |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Virtual Channel 39 |  |  | Virtual Channel 40 |  |  |  | Virtual Channel 41 |  |  |  |  |  | Virtual Channel 42 |  |  |
| PID | Value |  | PID | Value |  |  | PID |  | Value |  |  |  | PID |  | Value |
| Video | 390 |  | Video | 400 |  |  | Video |  | 410 |  |  |  | Video |  | 420 |
| Audio 1 | 391 |  | Audio 1 | 401 |  |  | Audio 1 |  | 411 |  |  |  | Audio 1 |  | 421 |
| Audio 2 | 392 |  | Audio 2 | 402 |  |  | Audio 2 |  | 412 |  |  |  | Audio 2 |  | 422 |
| DPI | 395 |  | DPI | 405 |  |  | DPI |  | 415 |  |  |  | DPI |  | 425 |
| DTMF Start DTMF End | 121* 121# |  | DTMF Start DTMF End | 121* 121# |  |  | DTMF Start DTMF End |  | 121* 121# |  |  |  | DTMF Start DTMF End |  | 121* 121# |
| Oxygen East |  |  | Oxygen West |  |  |  | Olympic Channel |  |  |  |  |  | Universo East |  |  |
| Virtual Channel 43 |  |  | Virtual Channel 44 |  |  |  | Virtual Channel 45 |  |  |  |  |  | Virtual Channel 46 |  |  |
| PID | Value |  | PID | Value |  |  | PID |  | Value |  |  |  | PID |  | Value |
| Video | 430 |  | Video | 440 |  |  | Video |  | 450 |  |  |  | Video |  | 460 |
| Audio 1 | 431 |  | Audio 1 | 441 |  |  | Audio 1 |  | 451 |  |  |  | Audio 1 |  | 461 |
| Audio 2 | 432 |  | Audio 2 | 442 |  |  | Audio 2 |  | 452 |  |  |  | Audio 2 |  | 462 |
| DPI | 435 |  | DPI | 445 |  |  | DPI |  | 455 |  |  |  | DPI |  | 465 |
| DTMF Start DTMF End | 121* 121# |  | DTMF Start DTMF End | 121* 121# |  |  | DTMF Start DTMF End |  | 121* 121# |  |  |  | DTMF Start DTMF End |  | 121* 121# |
| Universo West |  |  | Alt HD |  |  |  | USA East SD |  |  |  |  |  | Telemundo East SD |  |  |
| Virtual Channel 47 |  |  | Virtual Channel 48 |  |  | Virtual Channel 49 |  |  |  |  |  | Virtual Channel 66 |  |  |  |
| PID | Value |  | PID | Value |  | PID |  | Value |  |  |  | PID |  | Value |  |
| Video | 470 |  | Video | 480 |  | Video |  | 490 |  |  |  | Video |  | 660 |  |
| Audio 1 | 471 |  | Audio 1 | 481 |  | Audio 1 |  | 491 |  |  |  | Audio 1 |  | 661 |  |
| Audio 2 | 472 |  | Audio 2 | 482 |  | Audio 2 |  | 492 |  |  |  | Audio 2 |  | 662 |  |
| DPI | 475 |  | DPI | 485 |  | DPI |  | 495 |  |  |  | DPI |  | 665 |  |
| DTMF Start DTMF End | 121* 121# |  | DTMF Start DTMF End | 121* 121# |  | DTMF Start DTMF End |  | 121* 121# |  |  |  | DTMF Start DTMF End |  | 121* 121# |  |